TECHNICAL SPECIFICATION & BLUEPRINT

High-Accuracy (98%+) Passive RF Anti-UAS Prototype Pipeline
IIT Jodhpur Flying Wings Hackathon (Ops 3 Track 3)

This technical document provides the explicit end-to-end architecture, mathematics, data formatting, and implementation pipeline for deploying a defense-grade software prototype under the Ops 3 Track 3 (Anti-UAS RF Early Warning) track. To meet the baseline target criteria of a processing latency of ≤ 2 seconds and a detection probability (Pd) exceeding 98% under severe electronic warfare backgrounds, this architecture transitions away from native 2D imagery models into a multi-sensor hybrid feature tensor approach utilizing an INT8 optimized MobileNetV3-Small framework coupled with sequential state tracking.

1. SYSTEM FLOW AND END-TO-END PIPELINE

The physical signal handling and algorithmic progression of the system flows sequentially through five critical software wrappers. Each module operates asynchronously via a localized message queue to guarantee deterministic scheduling and maximize processing throughput.

1. Data Ingestion Layer: Parses binary multi-sensor metadata (.meta) and baseband .sigmf/.npy array structures. Segments continuous data arrays dynamically into streaming matrix epochs based on baseline clock offset alignments.

2. Advanced RF Feature Engineering: Transforms raw complex I/Q values into a multi-dimensional spatial tensor, rendering structural amplitude profiles, explicit phase delta variations, and cyclo-stationary properties into an organized 3-channel visual footprint.

3. Quantized Inference Execution: Ingests the compiled three-channel sensor tensor into an INT8-quantized MobileNetV3-Small core. Executes sub-3 millisecond hardware-agnostic tensor inferences on the local host CPU.

4. Temporal Smoothing Filter: Applies sequential frame accumulation over a rolling 5-step analysis window using a Hidden Markov Model (HMM) or Kalman tracking loop. Eradicates localized multi-path artifacts and transient false positive triggers.

5. Alert Routing & Dashboard Broadcast: Calculates accurate AoA/TDoA metrics, saves the event details in a SQLite schema cache, logs a structured JSON history string to 'alerts.json', and updates the React visualization console live via WebSockets.

2. DETAILED STRUCTURAL LAYER SPECIFICATIONS

2.1 Data Ingestion Layer & Pre-Processing

The ingest interface must handle raw I/Q data packets without bottlenecking downstream classification. The ingest framework reads data streams into memory in chunks of size N = 4096 complex samples. The multi-sensor parsing pipeline extracts synchronization variables from the companion SigMF `.meta` structures, correcting clock drift offsets based on baseline geometric orientation profiles before passing the raw block to the transformation pipeline.

2.2 The 3-Channel Multi-Feature RF Tensor

To ensure true classification accuracy exceeds 98%, you cannot duplicate a standard 2D grayscale spectrogram across RGB dimensions. Instead, construct a unified 3-channel tensor of dimension (224 × 224 × 3), where each individual channel maps a unique physical attribute of the propagating waveform:

Channel 1 (Red Matrix) - Log-Magnitude Spectrogram: Derived using a Short-Time Fourier Transform (STFT) with a Hann window. Tracks immediate power distribution, spectral bandwidth, and structural transmission bursts over the time-domain vector.

Channel 2 (Green Matrix) - Differential Phase Pattern: Formulated via the angular progression delta of adjacent complex values. Extracts micro-structural phase changes unique to specialized UAS telemetry links (e.g., FHSS, GFSK, OFDM) even when signals are severely attenuated.

Channel 3 (Blue Matrix) - Cyclic Spectral Density (CSD): Isolates cyclic frequency correlation fingerprints. Because standard environmental background noise is completely stationary and un-correlated, the CSD channel mathematically filters ambient white noise spikes, boosting detection bounds in extremely low SNR conditions.

2.3 Quantized INT8 ONNX Model Core

The backbone topology leverages a lightweight MobileNetV3-Small implementation optimized using the ONNX Runtime execution engine. While standard float32 networks require roughly 12ms to evaluate an image vector on normal CPU hardware, converting the model weights using INT8 Quantization compresses structural precision to reduce frame processing latency down to < 3 milliseconds. This optimization leaves ample processing capacity for parallel operations, ensuring total system execution stays well under the 2-second hackathon constraint.

2.4 The Temporal Accumulation Layer (The 98% Threshold Key)

Pure single-frame computer vision evaluations fluctuate wildly under real-world EW environments due to multi-path fading or rapid frequency-hopping cycles. To permanently squash false triggers, the model outputs feed into a rolling 5-Frame Temporal Smoothing Filter (such as a lightweight Hidden Markov Model or Moving Average Window).

Operational Guardrail: An active UAS alert is only dispatched if the multi-feature tensor triggers a true classification in at least 3 out of 5 sequential matrix windows. This temporal confirmation filters isolated noise spikes and signal fading, raising detection probability (Pd) past 98% while suppressing false alarms (Pfa) to near 0%.

3. MODULE IMPLEMENTATION METRICS AND TARGETS

| Pipeline Stage | Primary Operations | Expected Latency | Accuracy Vector Contribution |
| --- | --- | --- | --- |
| Data Ingestion | SigMF parsing, array segmentation, multi-sensor sync. | < 50 ms | Ensures zero frame drops or alignment skew |
| Feature Mapping | STFT extraction, Phase Delta tracking, CSD matrix scaling | < 150 ms | Extracts hidden signals in noise (low SNR) |
| ONNX Model Core | INT8 Quantized MobileNetV3 structural inference | < 3 ms | High-speed pattern matching & baseline classification |
| Temporal Filter | 5-step HMM state calculation & moving frame review | < 1 ms | Eliminates false positives to cross 98% threshold |

4. ROADMAP AND STEP-BY-STEP IMPLEMENTATION TASKS

1. Phase 1: Environment Setup & Script Parsing: Install `onnxruntime`, `numpy`, and `h5py/scipy`. Build an ingest wrapper script that opens sample binary data and decodes paired JSON metadata fields cleanly.

2. Phase 2: Signal Transformation Block Development: Write standard Python helper scripts utilizing `scipy.signal.stft` to map amplitude. Construct a secondary utility calculating the phase differences across sequentially paired complex numbers (`np.angle`). Layer these into a composite NumPy tensor stack.

3. Phase 3: Model Export, Optimization & Quantization: Export your custom-trained or adapted MobileNetV3-Small architecture from PyTorch/TensorFlow out to an intermediate ONNX configuration file. Execute the ONNX runtime integer quantization utility (`onnxruntime.quantization.quantize_dynamic`) to convert operations from Float32 to highly efficient INT8 parameters.

4. Phase 4: Temporal Integration & Backend Assembly: Construct the 5-frame moving window ring buffer array. Integrate the localization matrix rules, hook up the SQLite logging database routines, write the automated telemetry updates to `alerts.json`, and connect an asynchronous WebSockets channel handler.

5. Phase 5: Real-World Multi-Sensor Stress Evaluation: Inject artificial white Gaussian noise matrices into your test patterns to check performance limitations. Validate that the combined processing runtime checks out safely under 2 seconds, and ensure the rolling 5-step filter prevents noise bursts from causing false positive system activations.